Water Resource Analysis

By:

Nathaniel Holguin

Individual Analytical Analysis

Group Name: HCC26

Group Number: 8

Class Section: Thursday (001)

Professor: David Willy

Submitted towards partial fulfillment of the requirements for

Mechanical Engineering Design 1 Capstone C

– 11/23/2025

Introduction:

There are many things that need to be taken into account to develop a water turbine system into an existing dam. One of these that might be the most important is the actual water. The most important parts of the water that affect the power generated are the water flow and head that variate a lot over the course of the year. Another part of the water that will be of focus for the group is the water quality. This will entail all of the pollution from stormwater runoff, urbanization, and agricultural activities. The water quality could affect wildlife of the surrounding aquatic ecosystem. The water over the last couple years will be analyzed to see and understand fully how they might affect a turbine system setup at the John C. Stennis Dam.

Outflow Throughout the Year:

It is known that the outflow can vary a lot over the course of the year so monthly average data was collected from 1900-2014 excluding a couple of years due to missing data [1]. Taking the average flow rates for each month and putting them into a graph gives the following figure 1. The x axis number is the corresponding month of the year.

Figure 1: Monthly Average Outflow

As seen in figure 1 the flow at the dam follows a gradual path, having a high December through March and then dipping low May through November. This is due to clear seasonal trends where higher rainfall occurs in the winter and spring months and slows down in the summer and early fall months where precipitation is lower.

This gives the team a good outline to understand how outflow will change throughout the year and how the team might manage that. Even using a turbine that is attuned to capture low flow or a variety of flow, the energy produced during the low season would be insufficient and need a way to counteract it. Some options might include having more turbines run in these months, though that would be a superficial way to keep energy output up. Another option could be being operationally flexible, running as a run of water facility where power is only generated if there is adequate water supply. This might be able to be done seeing as the John C. Stennis Dam has gates that could be used to mitigate how much water flow goes through the system. A similar method to keep up energy through the low season would be to instead of saving water, saving power in a battery of some sorts so that ideally there would always be enough to distribute. There is only so much that can be done about low flow rates though, so in times of low flow the best option might just be to turn off some turbines and save some energy and environmental stability while letting a portion of the turbines do all the work. In this scenario other energy sources would be supplemented so that users would not be affected by seasonal outflow variations.

To understand the total outflow in a given year, data was collected for an average of every day of the year from 1900 to 2014 [1]. This data was used to make a outflow duration curve as seen below in figure 2.

Figure 2: Outflow Duration Curve

The outflow duration curve helps visualize how often outflow will be at different levels so teammates can better understand this topic. As seen on the curve, about 80% of the time outflow will be at least 2000cfs and only 20% of the time will it reach above 12000cfs.

Water Drop Variation:

Although outflow is more important, the difference between headwater and tailwater or water drop affects the energy output as well. Below in table 1 is data collected for 9 years [1] of head and tail water.

Table 1: Head and Tail Water

Thankfully these values do not have as much seasonal variation as outflow, so the energy output will not be affected too greatly. The total difference in water height at a monthly average sit around 24 to 27 feet, only having about a 3 foot variation. This low variation does not impact the energy output even a percentile as much as the outflow so in turn it will be neglected to deal with the much more pressing issue.

Conclusion:

With the data collected and analyzed it shows that the spring season gets the highest outflow where it will likely exceed 10000cfs and the winter/fall season gets the lowest outflow where it will likely fall below 4000cfs. This will likely be the main contributing factor to power generation due to the low seen difference in head values. With only a 3 foot variation, outflow ranging over 10000cfs will cause a much larger disturbance in power levels. This information will help the team push forward in turbine development at the John C. Stennis Dam. It allows the group to access the problem accurately and decide a plan moving forward. Ideal options to try and collect consistent power through the year involve saving power from stronger generation months. This could be done by focusing on either the more mechanical side or the more electric side. Having a  way to store power so that it can be used later or storing water so that it can generate power later would be likely routes moving forward.

References:

[1] U.S. Geological Survey, “Tombigbee River at John C Stennis Lock & Dam (USGS 02441390) - Real-Time Data,” USGS National Water Information System, 2025. [Online]. Available: https://waterdata.usgs.gov/ms/nwis/uv?site_no=02441390​

[2] U.S. Army Corps of Engineers, “Tombigbee River–John C. Stennis Lock & Dam–Water Control Data,” USACE Reservoir Control, 2025. [Online]. Available: https://rivergages.mvr.usace.army.mil​

[3] NOAA, “National Water Model–Tombigbee River Stennis Lock & Dam,” NOAA National Water Model Data, 2025. [Online]. Available: https://water.noaa.gov/​

[4] Mississippi Department of Environmental Quality, “Surface Water Quality in Mississippi: Tombigbee Basin,” MDEQ Water Quality Assessment Report, 2025. [Online]. Available: https://www.mdeq.ms.gov/​

[5] USGS, “Annual Water Data Reports: Tombigbee River at Stennis Lock & Dam,” USGS Water Data for Mississippi, 2025. [Online]. Available: https://waterdata.usgs.gov/nwis/annual/?site_no=02441390​

[6] Concordia University Library, “IEEE citation style - Citing,” IEEE Reference Guide, 2025. [Online]. Available: https://library.concordia.ca/help/citing/ieee.php​

[7] NJIT Libraries, “Getting started with IEEE referencing,” IEEE Citation Guide, 2018. [Online]. Available: https://researchguides.njit.edu/ieee-citation/ieeereferencing​

Appendix:

Table 2: Monthly Flow Rates (cfs)